SMART PARKING CONTROLLER

RTL Design and Verification using Verilog

Maincrafts Technology – 2-Day Skill Certification Program

# 1. Project Overview

The Smart Parking Controller is a digital RTL design that monitors vehicle entry and exit and keeps track of occupied and available parking slots. The design uses an 8-vehicle parking capacity and provides status signals for parking-full, entry permission, and exit permission.

# 2. Objective

To design and verify a Verilog RTL-based Smart Parking Controller that counts occupied parking slots, detects vehicle entry and exit, indicates available slots, and prevents entry when the parking area reaches its configured capacity.

# 3. Functional Requirements

Reset the occupied count to zero.

Increase the count for a valid vehicle entry.

Decrease the count for a valid vehicle exit.

Prevent the count from exceeding the capacity.

Indicate available slots.

Indicate when parking is full.

Provide entry and exit permission signals.

# 4. Input and Output Signals

| Signal | Type | Description |
| --- | --- | --- |
| clk | Input | Clock signal |
| reset | Input | Asynchronous reset |
| vehicle_entry | Input | Vehicle entry request |
| vehicle_exit | Input | Vehicle exit request |
| occupied_count | Output | Number of occupied slots |
| slots_available | Output | Number of free slots |
| parking_full | Output | High when capacity is reached |
| entry_allowed | Output | High when a slot is available |
| exit_allowed | Output | High when at least one vehicle is present |

# 5. System Working

After reset, the occupied count is zero. A valid entry increments the count when capacity is available. A valid exit decrements the count when at least one vehicle is present. Available slots are calculated as capacity minus occupied count.

# 6. RTL Design

The controller is implemented in Verilog RTL. Sequential logic is triggered by the rising edge of the clock or reset. The configured parking capacity is 8 vehicles.

# 7. Testbench and Verification

The testbench verifies reset, vehicle entries, full-capacity operation, an entry request while full, vehicle exit, and entry after a slot becomes available.

Reset: occupied_count = 0.

Vehicle entry: count increases by 1.

Full condition: count reaches 8 and parking_full becomes 1.

Entry while full: count remains 8.

Vehicle exit: count decreases by 1.

Entry after exit: the freed slot can be used.

# 8. Simulation Result

The RTL and testbench were successfully compiled and simulated using EDA Playground with Icarus Verilog. A $monitor statement was used to observe time, entry, exit, occupied count, available slots, full status, and entry permission.

# 9. Expected Working

| Test Case | Condition | Expected Result |
| --- | --- | --- |
| Reset | reset = 1 | Count becomes 0 |
| Entry | vehicle_entry = 1 | Count increases by 1 |
| Exit | vehicle_exit = 1 | Count decreases by 1 |
| Full | count = 8 | parking_full = 1; entry blocked |
| Entry while full | entry at count 8 | Count remains 8 |
| Exit from full | exit at count 8 | Count becomes 7 |

# 10. Tools Used

Verilog HDL

EDA Playground

Icarus Verilog

GitHub

# 11. Project Files

design.sv – RTL design

tb_smart_parking.sv – Testbench

simulation_output.png – Simulation evidence

README.md – Project documentation

# 12. Result

The Smart Parking Controller was successfully compiled and simulated. The testbench verified vehicle counting, capacity monitoring, entry blocking at full capacity, and vehicle exit behavior.

# 13. Conclusion

The project demonstrates the design and verification of a basic digital Smart Parking Controller using Verilog RTL. The controller maintains occupancy within the configured capacity and provides status and permission signals.